ИНСТРУКЦИЯ

по техники безопасности для обучающихся при работе на токарном станке по дереву

1. Общие требования к безопасности

1.1. Данная инструкция обязательна для всех обучающихся, выполняющих работы на токарном станке по дереву.

1.2. Обучающиеся должны знать, что в ходе выполнения работ на токарном станке по дереву чаще всего возникают такие травмы:

- ранение глаз отлетающей стружкой;

- ранение рук при их прикосновении к обрабатываемой детали;

- ранение рук неправильном обращении с резцом;

- ранение осколками плохо склеенной, косослойной, суковатой древесины

2. Требования к безопасности перед началом работы

Перед началом работы обучающиеся обязаны:

- правильно надеть спецодежду и головной убор (застегнуть обшлага рукавов на пуговицы; спрятать волосы под берет или косынку, завязанную без свисающих концов);

- проверить надежность крепления защитного  кожуха ременной передачи;

- проверить надежность крепления защитного заземления (зануления) к корпусу станка;

- убрать со станка все посторонние предметы и  инструменты, разложить их на установленные места;

- проверить отсутствие в заготовке сучков и трещин;

- обтесать заготовку до нужной формы, после чего надежно закрепить ее во вращающихся центрах на станке;

- установить подручник с зазором 2-3 мм от обрабатываемой детали и закрепить его на высоте центровой линии заготовки;

- проверить исправность режущего инструмента и правильность его заточки;

- проверить работу станка на холостом ходу, а также исправность пусковой коробки путем включения и выключения кнопок;

- надеть защитные очки.

3. Требования к безопасности во время работы

3.1. Во время работы обучающиеся должны соблюдать следующие требования:

- производить подачу режущего инструмента на материал только после того, как рабочий вал наберет полную частоту вращения;

- подавать инструмент плавно, без сильного нажима;

- своевременно подвигать подручник к обрабатываемой детали, не допуская увеличения зазора;

- измерять обрабатываемую деталь только после полной остановки ее вращения.

3.2. Во избежание травм во время работы на станке обучающимся запрещается:

- наклонять головы близко к станку;

- принимать и передавать предметы через работающий станок;

- останавливать станок путем торможения рукой обрабатываемой детали;

- отходить от станка не выключив его.

4. Требования к безопасности по окончании работ

4.1. После окончания работ обучающиеся обязаны:

- выключить станок;

- уложить на свои места инструменты;

- удалить со станка стружку при помощи щетки;

- очистить спецодежду;

- сдать станок учителю.

4.2. Не допускается сдувать стружку ртом или сметать рукой.

INSTRUCTIONS

safety for students at work on a lathe for wood

1. General safety requirements

1.1. These instructions compulsory for all students who perform work on a lathe for wood.

1.2. Students should be aware that in the course of work on the lathe for wood often arise such injuries:

• eye injury chips flying off;

• wounded hands when touching the work piece;

• wounded hands mishandled cutter;

• wounded by a poorly made, slanting layers, knotted wood

2. Safety requirements before starting work

Before you begin, students must:

• the right to wear protective clothing and a hat (to fasten the cuffs of sleeves with buttons, hide the hair under the beret or a scarf tied without dangling ends);

• check the reliability of mounting guard belt drive;

• check the reliability of fastening the protective earth (neutral) to the body of the machine;

• remove from the machine any objects and tools, put them on a specified place;

• check for the blank knots and cracks;

• to trim the work piece to the desired shape, and then secure it in rotating centers of the machine;

• hand device set with a gap of 2-3 mm from the work piece and secure it at the height of the center line of the work piece;

• сheck the accuracy of the cutting tool and its correct sharpening;

• check the operation of the machine at idle, and the serviceability of the starting box by switching on and off buttons;

• wear safety goggles.

3. Safety requirements during operation

3.1. During operation, students must meet the following requirements:

• produce feed material for the cutting tool only after the operating shaft reaches full speed;

• apply tools smoothly, without strong pressure;

• timely hand device  to move the work piece, not allowing the increase of the gap;

• measure the workpiece only after a full stop of its rotation.

3.2. To avoid injury while working on the machine, students are prohibited:

• tilt the head close to the machine;

• send and receive items through the machine running;

• stop the machine by hand brake of the workpiece;

• move away from the machine without turning it off.

4. Security requirements for completion of work

4.1. After completion of the work students are required to:

• turn off the machine;

• put into place the tools;

• remove chips from the machine with a brush;

• clean clothing;

• deliver to the teacher machine.

4.2. It is not allowed to blow off chips mouth or sweep hand.

ИНСТРУКЦИЯ

по технике безопасности для обучающихся при ручной обработке древесины

1. Общие требования к безопасности

1.1. Данная инструкция обязательна для всех обучающихся, выполняющихся, выполняющих работы, которые связаны с ручной обработкой древесины.

1.2. Обучающиеся должны знать, что в ходе работ, связанных с ручной обработкой древесины, чаще всего имеет место:

- травмы рук при работе неисправным инструментом;

- травмы рук при запиливании без применения необходимых приспособлений.

2. Требования к безопасности перед началом работ

Перед началом работы обучающиеся обязаны:

- надеть спецодежду (фартук с нарукавниками или халат) и головной убор (берет или косынку);

- тщательно подобрать волосы и заправить концы косынки;

- проверить наличие инвентаря (сиденья, щетки-сметки, совка) и исправность верстака (зажимных коробок, упора для пиления, зажимных клиньев, приспособления для чертежа);

- разложить на верстаке инструменты индивидуального пользования в порядке, установленном учителем;

- удалить с верстака все лишние инструменты и приспособления.

3. Требования к безопасности во время работ

3.1. В  процессе работы обучающиеся обязаны:

- использовать только исправный, хорошо налаженный и наточенный инструмент;

- надежно закрепить обрабатываемый материал (древесину) в зажимах верстака;

- использовать инструмент только по назначению;

- следить за правильностью выполнения различных способов и приемов обработки древесины;

- своевременно возвращать учителю инструмент общего пользования;

- очищать струги (рубанок, шерхебель, фуганок) от стружек деревянными клиньями;

- в случае порчи инструмента немедленно заменять его.

3.2. При ручной обработке древесины обучающимся следует:

- работать лучковой пилой только после того, как они убедятся, что полотно хорошо разведено и надежно закреплено в шаховках, а шнур обеспечивает необходимое его натяжение;

- выполнять технологические операции (пиление, обтесывание, долбление, сверление, соединение деталей) в специально установленных местах на верстаке, используя для этого различные приспособления;

- работать строгальными инструментами, имеющими исправный рожок (шерхебель, рубанок, фуганок) или вывеску (зензубель, калевка, галтель), а также округлую и гладкую заднюю часть колодки;

- приготавливать и разогревать клей только под наблюдением учителя в изолированном от мастерской и хорошо вентилируемом помещении;

- применять при запиливании направитель для опоры  полотна инструмента.

3.3. Обучающимся запрещается:

- допускать захламление верстака отходами и стружками;

- отвлекаться во время работы;

- пользоваться в деревообрабатывающей мастерской открытым огнем и электрообогревателями;

- сдувать стружку ртом и сметать ее рукой.

4. Требования к безопасности по окончании работ

После окончания работ обучающееся обязаны:

- сдать дежурному или учителю остатки материалов и незаконченные изделия;

- проверить состояние инструментов и положить их в порядке, установленном учителем;

- убрать свое рабочее место, пользуясь щеткой-сметкой;

- проверить наличие и состояние клиньев на верстаке;

завинтить до установленного зазора (не более 2-5мм) зажимные коробки (заднюю и переднюю);

- очистить спецодежду;

- выходить из кабинета только с разрешения учителя.

INSTRUCTIONS

safety for students for manual processing of wood

1. General safety requirements

1.1. These instructions compulsory for all students, perform, performing works that are associated with manual processing of wood.

1. 2. Students should be aware that in the course of work associated with manual processing of wood, often occurs:

• hand injury while working defective tool;

• hand injury when sawing without the necessary adaptations.

2. Safety requirements before starting work

Before you begin, students must:

• wear protective clothing (apron with over sleeves or robe) and a hat (beret or scarf);

• carefully choose the hair and tuck the ends of scarves;

• check the availability of equipment (seats, Brushes, shovels) and serviceability workbench (clamping boxes, stops for cutting, clamping wedges, tools for drawing);

• spread out on a workbench tools for individual use in the manner prescribed by the teacher;

• remove from the bench all the extra tools and accessories.

3. Requirements for safety during work

3.1. In the process, students must:

• use only properly functioning, well-organized and sharpened tools;

• securely attach the processed material (wood) in the clamps bench;

• use the tool only for its intended purpose;

• to monitor the correct implementation of the various methods and techniques of wood processing;

• promptly return the master tool for public use;

• clean the struga (planer jack-plane, jointer) from shavings wooden wedges;

• in the case of damage to the instrument immediately replace it.

3.2. In manual woodworking students should:

• work luchkova saw only after they are satisfied that the blade is well bred and is securely anchored in shahovkah, and provides the necessary line its tension;

• carry out technological operations (sawing,  scabbling, chiseling, drilling, joining of parts) in a specially designated locations on a workbench, using a variety of devices;

• work planning tools having defective horn (jack-plane, planer, jointer) or sign (zenzubel, fillister, fillet), as well as round and smooth the back of the pad;

• to prepare and warm up the adhesive only under the supervision of a teacher in an isolated workshop and well-ventilated area;

• apply at sawing guide for supporting the web tool.

3.3. Students are prohibited:

• avoid cluttering workbench waste and wood shavings;

• distracted while working;

• use in the woodworking shop open fire and electric heaters;

• blow chips and sour mouth with her hand.

4. Security requirements for completion of work

After graduating student work must:

• deliver to the teacher on duty or the remnants of materials and unfinished products;

• check the condition of the tools and put them in the order established by the teacher;

• remove their workplace, using the Brushes;

• check the presence and condition of the wedges on the bench;

screw to set the gap (not more than 2-5mm) clamping box (front and rear);

• clean clothing;

• out of the office only with the permission of the teacher.